Microprocessor and Computer Architecture

UE24CS251B

4th Semester, Academic Year 2025-26

Date:

| Name: Harshit Chandak | SRN: PES2UG24CS185 | Section C |
| --- | --- | --- |

LAB  #____4_______            Program Number: ____1___

Title of the Program

Write an ALP to perform Convolution using MLA instruction (Addition of multiplication of respective numbers of loc A and loc B).

- Typed ARM Assembly Code

- Output Screen Shot

( Screenshot including Register Window,Output Window and Code Window)

.DATA

A: .WORD 10,20,30,40,50

B: .WORD 10,20,30,40,50

.TEXT

LDR R0,=A

LDR R1,=B

MOV R2,#0

MOV R5,#0

LOOP:

LDR R3,[R0],#4

LDR R4,[R1],#4

MLA R5,R3,R4,R5

ADD R2,R2,#1

CMP R2,#5

BNE LOOP

SWI 0x11

.END

Microprocessor and Computer Architecture

UE24CS251B

4th Semester, Academic Year 2025-26

Date:

| Name: Harshit Chandak | SRN: PES2UG24CS185 | Section C |
| --- | --- | --- |

LAB  #____4_______            Program Number: ____2___

Title of the Program

Write a program in ARM7TDMI-ISA to generate a diagonal matrix.

; Note: do not read the matrix elements.

Let the Diagonal Value be 2

- Typed ARM Assembly Code

- Output Screen Shot

(One Screenshot including Register Window,Output Window and Code Window,Memory Window)

.DATA

A: .WORD 0,0,0,0,0,0,0,0,0

.TEXT

MOV R0,#2

MOV R1,#0

LDR R2,=A

LOOP:

STR R0,[R2],#16

ADD R1,R1,#1

CMP R1,#3

BNE LOOP

SWI 0x11

.END

Microprocessor and Computer Architecture

UE24CS251B

4th Semester, Academic Year 2025-26

Date:

| Name: Harshit Chandak | SRN: PES2UG24CS185 | Section C |
| --- | --- | --- |

LAB  #____4_______            Program Number: ____3___

Title of the Program

Write a program in ARM7TDMI-ISA to transfer a block of 16 words stored at memory location X to memory location Y using Load Multiple and Store Multiple instructions. The rate of transfer is 24 bytes

- ARM Assembly Code

- Output Screen Shot

(One Screenshot including Register Window,Memory Window and Code Window)

.DATA

A: .WORD 1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,16

B: .WORD 0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0

.TEXT

MOV R0,#16

MOV R1,#0

LDR R3,=A

LDR R4,=B

LOOP:

MOV R2,#0

INNERLOOP:

LDR R5,[R3],#4

STR R5,[R4],#4

SUB R0,R0,#1

ADD R2,R2,#1

CMP R2,#6

BNE INNERLOOP

ADD R1,R1,#1

CMP R1,#2

BNE LOOP

REMAINING:

LDR R5,[R3],#4

STR R5,[R4],#4

SUB R0,R0,#1

CMP R0,#0

BNE REMAINING

BEQ EXIT

EXIT:

SWI 0x11

.END

2 ND WAY

.DATA

A: .WORD 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16

B: .WORD 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0

.TEXT

LDR R0, =A

LDR R1, =B

MOV R2, #16

LOOP:

LDMIA R0!, {R3-R8}

STMIA R1!, {R3-R8}

SUB R2, R2, #6

CMP R2, #6

BLT REMAINDER

B LOOP

REMAINDER:

CMP R2, #0

BEQ EXIT

LDMIA R0!, {R3}

STMIA R1!, {R3}

SUB R2, R2, #1

B REMAINDER

EXIT:

SWI 0X11

Microprocessor and Computer Architecture

UE22CS251B

4th Semester, Academic Year 2023-24

Date:

| Name:Harshit Chandak | SRN:PES2UG24CS185 | Section C |
| --- | --- | --- |

LAB  #____4_______            Program Number: ____4___

Title of the Program

Write an ARM ALP to perform element-wise addition of two 3×3 matrices using indexed addressing and nested loops.

Requirements

- Matrices A and B each contain 9 elements (3 rows × 3 columns)

- Matrix C should store the sum of corresponding elements

- Use nested loops to traverse rows and columns

- Use the MLA (Multiply Accumulate) instruction to compute the memory offset

- Each matrix element is a 32-bit word

- ARM Assembly Code

- Output Screen Shot

(One Screenshot including Register Window,Memory Window and Code Window)

.DATA

A: .WORD 1,2,3,4,5,6,7,8,9

B: .WORD 1,2,3,4,5,6,7,8,9

C: .WORD 0,0,0,0,0,0,0,0,0

.TEXT

LDR R0,=A

LDR R1,=B

LDR R2,=C

MOV R5,#0

MOV R7,#1

LOOP:

LDR R3,[R0]

LDR R4,[R1]

MLA R6,R3,R7,R4

STR R6,[R2]

ADD R0,R0,#4

ADD R1,R1,#4

ADD R2,R2,#4

ADD R5,R5,#1

CMP R5,#9

BNE LOOP

SWI 0x11

.END

Microprocessor and Computer Architecture

UE22CS251B

4th Semester, Academic Year 2023-24

Date:

| Name: Harshit Chandak | SRN: PES2UG24CS185 | Section C |
| --- | --- | --- |

LAB #____4_______            Assignment Question 1

Title of the Program

Write an ARM7TDMI ALP to check whether a given 32-bit number has odd or even parity (number of 1s).
It stores 00 in R7 if parity is EVEN and 01 in R7 if parity is ODD

- ARM Assembly Code

- Output Screen Shots

(Two Screenshot for each case including Register Window,Memory Window and Code Window)

.DATA

NUM: .WORD 4

PARITY: .WORD 0

.TEXT

LDR R0,=NUM

LDR R1,[R0]

LDR R7,=PARITY

MOV R8,#00

MOV R9,#01

MOV R2,#0

MOV R3,#0

LOOP:

TST R1,#1

ADDNE R2,R2,#1

MOV R1,R1,LSR #1

ADD R3,R3,#1

CMP R3,#32

BNE LOOP

TST R2,#1

STRNE R9,[R7]

STREQ R8,[R7]

.END

Microprocessor and Computer Architecture

UE22CS251B

4th Semester, Academic Year 2023-24

Date:

| Name: Harshit Chandak | SRN: PES2UG24CS185 | Section C |
| --- | --- | --- |

LAB #____3_______

Assignment Question 2

Title of the Program

Write an ALP to perform Convolution using MUL instruction (Addition of multiplication of respective numbers of loc A and loc B)

- ARM Assembly Code

- Output Screen Shots

(One Screenshot including Register Window,Memory Window and Code Window)

.DATA

A: .WORD 10,20,30,40,50

B: .WORD 10,20,30,40,50

.TEXT

LDR R0,=A

LDR R1,=B

MOV R2,#0

MOV R5,#0

MOV R6,#0

LOOP:

LDR R3,[R0],#4

LDR R4,[R1],#4

MUL R5,R3,R4

ADD R6,R6,R5

ADD R2,R2,#1

CMP R2,#5

BNE LOOP

SWI 0x11

.END

Disclaimer:

- The programs and output submitted is duly written, verified and executed by me.

- I have not copied from any of my peers nor from the external resource such as internet.

- If found plagiarized, I will abide with the disciplinary action of the University.

Signature:

Name:

SRN:

Section:

Date: