Microprocessor and Computer Architecture

UE24CS251B

4th Semester, Academic Year 2025-26

Date:

| Name: Harshit Chandak | SRN:PES2UG24CS185 | Section C |
| --- | --- | --- |

LAB  #____2_______            Program Number: ____1___

Title of the Program

Based on value of number in r0,

- store 1 in r1 if r0 is 0

- store 2 in r1 if r0 is positive

- store 3 if r0 is negative

- Typed ARM Assembly Code

- Three Output Screen Shots (one for each case )

( Screenshot including Register Window,Output Window and Code Window)

.DATA

NUM: .WORD 0

.TEXT

LDR R0,=NUM

LDR R0,[R0]

CMP R0,#0

BEQ ZERO

BGT POS

MOV R1,#3

B EXIT

ZERO:

MOV R1,#1

B EXIT

POS:

MOV R1,#2

B EXIT

EXIT: SWI 0x011

.END

Microprocessor and Computer Architecture

UE24CS251B

4th Semester, Academic Year 2025-26

Date:

| Name: Harshit Chandak | SRN: PES2UG24CS185 | Section C |
| --- | --- | --- |

LAB  #____2_______            Program Number: ____2___

Title of the Program

Write an ARM program that generates 75 × R0 using shift and add/subtract operations (no MUL instruction).

- ARM Assembly Code

- Output Screen Shots

(One Screenshot including Register Window,Output Window and Code Window)

.DATA

A: .WORD 1

.TEXT

LDR R0,=A

LDR R0,[R0]

ADD R1,R0,R0,LSL#6

ADD R2,R0,R0,LSL#3

ADD R3,R1,R2

ADD R4,R3,R0

SWI 0x011

.END

Microprocessor and Computer Architecture

UE24CS251B

4th Semester, Academic Year 2025-26

Date:

| Name: Harshit Chandak | SRN:PES2UG24CS185 | Section C |
| --- | --- | --- |

LAB  #____2_______            Program Number: ____3___

Title of the Program

To find GCD of two numbers using conditional instructions only.

- ARM Assembly Code

- Output Screen Shots

(One Screenshot including Register Window,Memory Window and Code Window)

.DATA

NUM1: .WORD 38

NUM2: .WORD 95

.TEXT

LDR R1,=NUM1

LDR R1,[R1]

LDR R2,=NUM2

LDR R2,[R2]

LOOP:

CMP R1,R2

SUBGT R1,R1,R2

SUBLT R2,R2,R1

BNE LOOP

BEQ EXIT

EXIT:

MOV R3,R1

SWI 0x011

.END

Microprocessor and Computer Architecture

UE22CS251B

4th Semester, Academic Year 2023-24

Date:

| Name: Harshit Chandak | SRN: PES2U24CS185 | Section |
| --- | --- | --- |

LAB  #____2_______            Program Number: ____4___

Title of the Program

Add 2 numbers from memory and store result in memory using byte addition

- ARM Assembly Code

- Output Screen Shots

(One Screenshot including Register Window,Memory Window and Code Window)

.DATA

NUM1: .BYTE 5

NUM2: .BYTE 30

NUM3: .BYTE 0

.TEXT

LDR R0,=NUM1

LDR R1,=NUM2

LDR R2,=NUM3

LDRB R0,[R0]

LDRB R1,[R1]

ADD R3,R0,R1

STRB R3,[R2]

SWI 0x011

.END

Microprocessor and Computer Architecture

UE22CS251B

4th Semester, Academic Year 2023-24

Date:

| Name: Harshit Chandak | SRN: PES2UG24CS185 | Section C |
| --- | --- | --- |

LAB  #____2_______            Assignment Question 1

Title of the Program

Write an ALP using ARM7TDMI to add only even numbers stored in memory location for a given set of numbers and store the sum in the memory location.

- ARM Assembly Code

- Output Screen Shots

(One Screenshot including Register Window,Memory Window and Code Window)

.DATA

A:   .WORD 1,2,3,4,5,6,7,8,9,10

SUM: .WORD 0

.TEXT

LDR R0,=A

MOV R1,#10

MOV R2,#0

LOOP:

LDR R3,[R0]

TST R3,#1

BNE SKIP

ADD R2,R2,R3

SKIP:

ADD R0,R0,#4

SUBS R1,R1,#1

BNE LOOP

LDR R4,=SUM

STR R2,[R4]

SWI 0x011

.END

Microprocessor and Computer Architecture

UE22CS251B

4th Semester, Academic Year 2023-24

Date:

| Name: Harshit Chandak | SRN:PES2UG24CS185 | Section C |
| --- | --- | --- |

LAB #____2_______

Assignment Question 2

Title of the Program

Write an ALP to evaluate the expression (A+B) + (13*B), where A and B are available in memory location. Store the final result in memory Location C

* Use LSL instruction for multiplication

- ARM Assembly Code

- Output Screen Shots

(One Screenshot including Register Window,Memory Window and Code Window)

.DATA

A: .WORD 9

B: .WORD 6

C: .WORD 0

.TEXT

LDR R0,=A

LDR R1,=B

LDR R6,=C

LDR R0,[R0]

LDR R1,[R1]

ADD R2,R0,R1

ADD R3,R1,R1,LSL#2

ADD R4,R3,R1,LSL#3

ADD R5,R2,R4

STR R5,[R6]

SWI 0x011

.END

Disclaimer:

- The programs and output submitted is duly written, verified and executed by me.

- I have not copied from any of my peers nor from the external resource such as internet.

- If found plagiarized, I will abide with the disciplinary action of the University.

Signature:

Name:

SRN:

Section:

Date: